Alternate code for Stepmeter

Adds ability to reset and increases chance that a step will be detected

from microbit import *

# Define a variable to record steps

steps = 0

while True:

# Check to see if a step has been taken.

# If so, display a smile and increase the number of steps by 1

if accelerometer.was_gesture('shake')or accelerometer.was_gesture('right')or    accelerometer.was_gesture('left')or accelerometer.was_gesture('up')or accelerometer.was_gesture('down'):

steps += 1

display.show(Image.HAPPY)

sleep(500)

display.clear()

# Check to see if button A has been pressed.

# If so, display the number of steps taken

elif button_a.is_pressed():

string_steps = str(steps)

display.show(string_steps)

sleep(2000)

display.clear()

elif button_b.is_pressed():

steps = 0

display.scroll('reset')

display.clear()

Alternate code for sparkles

Adds ability to turn off sparkles

from microbit import *

import random

images = [Image.HAPPY, Image.SILLY, Image.SKULL, Image.MEH,

Image.PACMAN, Image.HEART]

while True:

if button_a.was_pressed() and button_b.was_pressed():

display.clear()

elif accelerometer.was_gesture('face up'):

while True:

x = random.randint(0, 4)

y = random.randint(0, 4)

brightness = random.randint(0, 9)

display.set_pixel(x, y, brightness)

if button_a.was_pressed() and button_b.was_pressed():

display.clear()

break

if button_a.was_pressed():

display.show(random.choice(images))

sleep(1000)

if button_b.was_pressed():

display.scroll("Hello", delay=100)

sleep(1000)

if accelerometer.was_gesture('shake'):

display.show(Image.ANGRY)

sleep(1000)